UNITED STATES ENVIRONMENTAL PROTECTION AGENCY

REGION 10

1200 Sixth Avenue, Suite 900

Seattle, Washington 98101-3140

OFFICE OF ENVIRONMENTAL CLEANUP

EMERGENCY RESPONSE UNIT

Site Specific Sampling Plan

Project Name: Young America Mine Tailings Impoundment Removal Assessment	Site ID: 10KN

Author: Amy Dahl	  Company: TechLaw, Inc.	  Date Completed: 6/24/2011

This Site Specific Sampling Plan (SSSP) is prepared and used in conjunction with the Quality Assurance Plan (QAP) for the Emergency Response Unit for collecting samples during this Removal Program project. The information contained herein is based on the information available at the time of preparation. As better information becomes available, this SSSP will be adjusted.

When inadequate time is available for preparing the SSSP in advance of the sampling event, a Field Sampling Form may be prepared on-site immediately prior to sampling. This full length version of the SSSP is written after the sampling event and the completed Field Sampling Form attached to it.

1. Approvals

| Name, Title | Telephone, Email, Address | Signature |
| --- | --- | --- |
| Kathy Parker On-Scene Coordinator | 206-553-0062, parker.kathy@epa.gov USEPA , M/S: ECL-116, 1200 Sixth Ave. Suite 900, Seattle, WA 98101 |  |
| Michael Boykin EMP alt Quality Assurance Coordinator | 206-553-6362, oykin.michael@epa.gov USEPA , M/S: ECL-116, 1200 Sixth Ave. Suite 900, Seattle, WA 98101 |  |

I. Project Management and Organization

2. Personnel and Roles involved in the project:

| Name | Telephone, Email, Company, Address | Project Role | Data Recipient |
| --- | --- | --- | --- |
| Kathy Parker | 206 553 0062, parker.kathy@epa.gov USEPA , M/S: ECL-116, 1200 Sixth Ave. Suite 900, Seattle, WA 98101 | On Scene Coordinator | Yes |
| Amy Dahl | 206-577-3050, adahl@techlawinc.com TechLaw, Inc., 101 Yesler Way, Suite 600, Seattle, WA 98104 | Author of SSSP, START Project Manager | Yes |
| Michael Boykin | 206 553 6362, boykin.michael@epa.gov USEPA , M/S: ECL-116, 1200 Sixth Ave. Suite 900, Seattle, WA 98101 | ERU Quality Assurance Coordinator | No |
| Paul Swift | 206-577-3058, pswift@techlawinc.com TechLaw, Inc., 101 Yesler Way, Suite 600, Seattle, WA 98104 | START Quality Assurance Reviewer | Yes |
| MEL | 360-871-8728 dodo.gerald@epa.gov USEPA Region 10 Lab 7411 Beach Drive East, Port Orchard, WA 98366 | Laboratory contact | No |

3. Physical Description and Site Contact Information:

| Site Name |  | Young America Mine Tailings Impoundment |  |  |
| --- | --- | --- | --- | --- |
| Site Location |  | SE¼SW¼ sec. 28, and NE¼NW¼ sec. 33, T38N R38E, about 2000 feet northeast of and 400 feet above Bossburg, WA |  |  |
| Property Size |  | 2 acres |  |  |
| Site Contact |  | Diana Broyles, diana.broyles@gmail.com | Phone Number: (509)684-4435 |  |
| Nearest Residents |  | on site | Direction: NA |  |
| Primary Land Uses Surrounding the Site |  | residential, recreational |  |  |

4. The proposed schedule of project work follows:

| Activity | Estimated Start Date | Estimated Completion Date | Comments |
| --- | --- | --- | --- |
| SSSP Review/Approval | 6/21/2011 | 6/30/2011 |  |
| Mobilize to / Demobilize from Site | 7/5/2011 | 7/8/2011 |  |
| Sample Collection | 7/6/2011 | 7/7/3011 |  |
| Laboratory Sample Receipt | 7/11/2011 | 7/12/2011 |  |
| Laboratory Analysis | 7/12/2011 | 8/12/2011 | Preliminary verbal data within 2 weeks, final data within 6 weeks of sample delivery |
| Data Validation | 8/15/2011 | 9/6/2011 |  |
| Draft Report | 7/25/2011 | 9/9/2011 |  |
| Final Report | 9/26/2011 | 9/30/2011 |  |

5. Historical and Background Information

Describe briefly what you know about the site that is relevant to sampling and analysis for this investigation.

The Young America Mine is located on the east bank of Lake Roosevelt and the Columbia River. It was mined for zinc, lead, silver, and gold and operated from 1897 through 1953. The mine itself is on BLM land but the tailings pile is located on private land subdivided into 4 parcels. The lateral extent of the tailings pile has been estimated at 1-2 acres. Washington DNR personnel performed site characterization work in September 2001, July 2003, and August 2006 and found 7000 ppm lead, 10,000 ppm zinc, 93 ppm copper, 63 ppm cadmium and 61 ppm arsenic concentrations in one sample of the tailings. They also photographed a real estate development utility trench through the tailings. The Confederated Tribes of the Colville Reservation has identified the mine as an area of concern. WDOE and EPA conducted a site visit on 6/16/2011 and identified large tailings piles on private property being sold for residential and recreation land. One recently installed drinking water well appears to be placed in the tailings impoundment. WDOE referred the site to EPA for assessment. This site is within the 150-mile stretch of the Upper Columbia River between the US-Canadian border and the Grand Coulee Dam currently undergoing CERCLA investigation. In an internal EPA meeting on 6/20/2011, it was agreed between the SAM, RPM and Removal Manager that the assessment would not be integrated. Any contamination that could not be addressed by the removal program would be referred to the site assessment or remedial programs.

6. Conceptual Site Model

Example: Contaminant:  Mercury

Transport Mechanism: vapor moving on air currents

Receptors: people living in the house

Contaminants: lead, arsenic, zinc, copper, cadmium Transport Mechanisms: human ingestion of drinking water, run-off by precipitation to the lake, ingestion of dust Receptors: Aquatic species in lake, migratory birds, residents and visitors to site, burrowing animals at site

7. Decision Statement

Examples: 1) Determine whether surface contamination exceeds the established action level;

2) Determine appropriate disposal options for contaminated materials.

The decision(s) to be made from this investigation is/are to: - Determine if concentrations of metals in soil and groundwater exceed applicable risk and disposal action levels - Determine the approximate volume of tailings that exceed applicable action levels

8. Action Level

State the analyte, concentration, and units for each selected action level. Describe the rationale for choosing each action level and its source (i.e. MTCA, PRG, ATSDR, etc.) Example: The action level for total mercury in soil is 6.7 mg/kg (from Regional Screening Level residential).

Soil: 1) EPA RAL for residential soil, 2) EPA RSL for residential soil, 3) Washington MTCA Soil Method A for unrestricted land use, 4) NOAA SQUIRT values for Inorganics in Soil (lowest of EPA Eco-SSLs, ORNL screening benchmarks, or EPA R5 Eco Screening levels), and 5) Washington MTCA ecological criteria. Groundwater: 1) EPA RAL for tapwater, 2) EPA RSL for tapwater, 3) EPA MCL for drinking water, and 4) Washington MTCA groundwater Method A Table. SPLP Leachates: these action levels are the same as the groundwater action levels plus the most conservative EPA Water Quality Criteria. TCLP Leachates: the action levels are the maximum concentration of contaminants for Toxicity Characteristics, excluding mercury (Table 7-1 in Chapter 7 of SW-846). If the analytical results for Total Metals are less than twenty times the TCLP threshold levels, they will be used in lieu of preparing TCLP leachates.

II. Data Acquisition and Measurement Objectives

9. Site Diagram and Sampling Areas

A Sampling Area is an area within in which a specific action will be performed.

Examples : 1) Each drum on the site is a Sampling Area;

2) Each section of sidewalk in front of the residence is a Sampling Area;

3) Each sampling grid section is a Sampling Area.

Source: http://www.dnr.wa.gov/Publications/ger_ic105_iaml_youngamerica.pdf

10. The Decision Rules

These can be written as logical If…, Then.. statements. Describe how the decisions will be made and how to address results falling within the error range of the action level. Examples: 1) In the Old Furnace Sampling Area, the soil in the area around the furnace structure will be excavated until sample analysis with XRF shows no mercury concentrations in surface soil above the lower limit of the error associated with the action level, 18.4 mg/kg. 2) If the concentrations of contaminants in a SA are less than the lower limit of the error associated with the action level, then the area may be characterized as not posing an unacceptable risk to human health or the environment and may be dismissed from additional RP activities. The area may be referred to other Federal, State or Local government agencies.

The following statement(s) describe the decision rules to apply to this investigation: If analytical results for target metal analytes in the samples exceed action levels, then the sample locations may be considered for potential further actions as determined by the OSC.

11. Information Needed for the Decision Rule

What information needs to be collected to make the decisions – this includes non-sampling info as well: action levels, climate history, direction of water flow, etc. Examples: Current and future on-site and off-site land use; wind direction, humidity and ambient temperature; contaminant concentrations in surface soil.

The following inputs to the decision are necessary to interpret the analytical results: - Endangered or threatened species affected by the site contaminants. - Site risk analysis based on animal and plant species using the site. - Direction and volume of surface water flow and ability of run-off and particulates in run-off to enter nearby water body.

12. Sampling and Analysis

For each SA, describe:

1. sampling pattern (random, targeted, scheme for composite)

2. number of samples, how many to be collected from where, and why

3. sample type (grab, composite)

4. matrix (air, water, soil)

5. analytes and analytical methods

6. name and locations of off-site laboratories, if applicable.

XRF Screening: - Center of each grid section - A step-out grid sampling approach will be utilized to delineate the lateral extent of metal contamination from the mine tailings. A 700’ x 500’ grid will be divided into 35 sections with 100’ spacing (see site diagram in Section 9). Starting in the approximate center of the grid, soil between approximately 1’-2’ below ground surface (below any sand/gravel cover) will be collected using a shovel into a sealable plastic bag, homogenized in the bag, and analyzed by XRF through the side of the bag. If contamination above action levels is detected by XRF in the perimeter cells of the grid, step-outs of 100’ will occur outward from the contaminated area until the perimeter is established. To determine the vertical extent, 1-3 targeted locations will be sampled at depth using a hand auger until clean soil is reached, as confirmed by XRF. Depth to clean soil will be recorded for using in volume calculation. - Grab - Soil - TAL metals by XRF - Field screening Soil Samples for laboratory analysis: - Highest XRF reading for lead - Of the samples collected for XRF screening, the three containing the highest concentration of lead will be submitted for laboratory analysis for the purposes of confirming the XRF results and determining SPLP and TCLP efficiency. No duplicate field sample will be submitted to the lab because the XRF screening results will provide sampling precision information. The lab will perform a laboratory duplicate analysis for one Total Metals sample. - Grab - Soil - 3 samples for TAL metals by ICP-AES and SPLP TAL metals by ICP-MS and 2 samples for TCLP metals by ICP-AES. (Mercury analyses will not be requested.) - MEL will perform the analyses. Groundwater Samples: - One sample from the drinking water well in the tailings pile and one background sample from the home on plat 7 - Two drinking water well samples, a field duplicate, and an equipment blank will be collected. - Grab - Groundwater - TAL metals by ICP-MS. (Mercury analyses will not be requested.) - MEL will perform the analyses.

13. Applicability of Data 	(place an X in front of the data categories needed, explain with comments)

Do the decisions to be made from the data require that the analytical data be:

1) definitive data, 2) screening data (with definitive confirmation) or 3) screening data (without definitive confirmation)?

X  A) Definitive data is analytical data of sufficient quality for final decision-making. To produce definitive data on-site or off-site, the field or lab analysis will have passed full Quality Control (QC) requirements (continuing calibration checks, Method Detection Limit (MDL) study, field duplicate samples, field blank, matrix spikes, lab duplicate samples, and other method-specific QC such as surrogates) AND the analyst will have passed a Precision and Recovery (PAR) study AND the instrument will have a valid Performance Evaluation sample on file. This category of data is suitable for: 1) enforcement purposes, 2) determination of extent of contamination, 3) disposal, 4) RP verification or 5) cleanup confirmation.

Comments: Well water, SPLP and TCLP of tailings, and Total Metals to compare to leachate results (these are the same three samples used to confirm XRF results).

X  B) Screening data with definitive confirmation is analytical data that may be used to support preliminary or intermediate decision-making until confirmed by definitive data. However, even after confirmation, this data is often not as precise as definitive data. To produce this category of data, the analyst will have passed a PAR study to determine analytical error AND 10% of the samples are split and analyzed by a method that produced definitive data with a minimum of three samples above the action level and three samples below it.

Comments: Confirm 3 XRF tailings and soil field samples at lab

X   C) Screening data is analytical data which has not been confirmed by definitive data. The QC requirements are limited to an MDL study and continuing calibration checks. This data can be used for making decisions: 1) in emergencies, 2) for health and safety screening, 3) to supplement other analytical data, 4) to determine where to collect samples, 5) for waste profiling, and 6) for preliminary identification of pollutants. This data is not of sufficient quality for final decision-making.

Comments:  XRF tailings and soil

14. Special Sampling or Analysis Directions

Describe any special directions for the planned sampling and analysis such as additional quality controls or sample preparation issues. Examples: 1) XRF and Lumex for sediment will be calibrated before each day of use and checked with a second source standard. 2) A field blank will be analyzed with each calibration to confirm the concentration of non-detection. 3) A Method Detection Limit determination will be performed prior to the start of analysis so that the lower quantitation limit can be determined. 4) If particle size is too large for accurate analyses, the samples will be ground prior to analysis. If the sample contains too much moisture for accurate analyses, the sample will be decanted and air dried prior to analysis.

- XRF for field screening will be calibrated before each day of use and checked with a second source standard. MDL determinations for each metal of interest will be performed prior to use on samples (i.e., either before field work or on-site). Every analyst operating the XRF will have performed PAR studies for each metal of interest prior to use on samples. - An equipment blank will be collected for the pump used for groundwater collection. - A rinsate blank will be collected for the shovel used to collect subsurface soil samples. - The same three tailing samples will be used for all tailings lab analyses in order to determine leachate efficiencies without introducing inter-sample variability.

15. Method Requirements

[Describe the restrictions to be considered in choosing an analytical method due to the need to meet specific regulations, policies, ARARs, and other analytical needs. Examples: 1) Methods must meet USEPA Drinking Water Program requirements. 2) Methods must achieve lower quantitation limits of less than 1/10 the action levels.3) Methods must be performed exactly as written without modification by the analytical laboratory.]

- Well water sample analysis methods must meet USEPA Drinking Water Program requirements. - All analyses must be by methods acceptable to WDOE. - Methods will be selected to achieve minimum quantitation limits of less than 1/10 the action levels. Any analyses with non-detect results above the appropriate action level may be targeted for reanalysis by a more sensitive method. Additional sample volume will be collected and stored until the need for re-analysis can be determined.

16. Sample Collection Information

[Describe any activities that will be performed related to sample collection]

The applicable sample collection Standard Operating Procedures (SOPs) or methods will be followed and include: 03-01-xx Field Documentation Procedures – Maintaining a Field Logbook 04-02-xx Packaging and Shipping Procedures – Environmental Samples 02-03-xx Field Procedures - Equipment Decontamination 07-03-xx Soil Sampling and Analysis Procedures – Surface/near-surface Soil Sampling 06-03-xx Groundwater Sampling/Monitoring and Analysis Procedures – Pre-Sampling Activities 06-04-xx Groundwater Sampling/Monitoring and Analysis Procedures – Sampling Activities TechLaw’s Innov-X XRF draft SOP ERU SOG 119B Analytical Data Management

17. Optimization of Sampling Plan (Maximizing Data Quality While Minimizing Time and Cost)

[Describe what choices were made to reduce cost of sampling while meeting the needed level of data quality. Example: The XRF will be used in situ whenever possible to achieve accurate results. Reproducibility and accuracy of in situ XRF analyses will be checked by collecting, air drying, analyzing and comparing five in situ samples at the start of sampling. Where interferences are suspected, steps will be taken to eliminate the interferences by mechanisms such as drying, grinding or sieving the samples or analyzing them using the Lumex with soil attachment.]

The XRF will be used for field screening of soil with only 3 soil samples submitted to laboratory for confirmation. This will decrease the cost of field labor and laboratory analyses and achieve real-time analytical data to quickly determine the lateral and vertical edges of contamination.

III. Assessment and Response

A Sample Plan Alteration Form (SPAF) will be used to describe project discrepancies (if any) that occur between planned project activities listed in the final SSSP and actual project work.  The completed SPAF will be approved by the OSC and QAC and appended to the original SSSP.

A Field Sampling Form (FSF) may be used to capture the sampling and analysis scheme for emergency responses in the field and then the FSF pages can be inserted into the appropriate areas of the final SSSP.

Corrective actions will be assessed by the sampling team and others involved in the sampling and a corrective action report describing the problem, solution, and recommendations will be forwarded to the OSC and the ERU QAC.

IV. Data Validation and Usability

The sample collection data will be entered into Scribe and Scribe will be used to print lab Chains of Custody.  Results of field and lab analyses will be entered into Scribe as they are received and uploaded to Scibe.net when the sampling and analysis has been completed.

18. Data Validation or Verification will be performed by:

ERU’s general recommendation on validation is that a minimum of CLP-equivalent stage IIA verification and validation be performed for every SSSP involving laboratory analyses. However, stage IIB is preferred if the lab can provide it. Dioxins should be validated at CLP-equivalent stage 4.

|  | Data Verification and Validation Stages |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Performed by: | I | IIA | IIB | III | IV | Verification | Other: |
| E and E QA Reviewer |  |  |  |  |  |  |  |
| TechLaw QA Reviewer |  |  | 100% (mod) |  | 10% (mod) |  |  |
| EPA Region 10 QA Office |  |  |  |  |  |  |  |
| MEL staff |  |  |  |  |  | 100% of MEL data |  |
| Other: |  |  |  |  |  |  |  |

The format for sample number identification is summarized in Table 1. Sample collection and analysis information is summarized in Table 2.

| Table 1 SAMPLE CODING Project Name: Young America Mine Tailings Impoundment Removal Assessment Site ID: 10KN |  |  |
| --- | --- | --- |
| SAMPLE NUMBER (1) |  |  |
| Digits | Description | Code (Example) |
| 1,2,3,4 | Year and Month Code |  |
| 5,6,7,8 | Consecutive Sample Number (grouped by SA as appropriate) |  |

| SAMPLE NAME / LOCATION ID (2) (Optional) |  |  |
| --- | --- | --- |
| 1,2 | Sampling Area | BG – Background DR – Drum LF – Landfill MW – Monitoring Well RS – Rinsate SI – Surface Impoundment TB – Trip Blank TK – Tank WL – Wetland WP – Waste Pile |
| 3,4 | Consecutive Sample Number | 01 – First sample of Sampling Area |
| 5,6 | Matrix Code | AR – Air GW – Groundwater PR – Product SB – Subsurface Soil SD – Sediment SS – Surface Soil SW – Surface Water QC – Quality Control WT – Water |
| 7,8 | Depth (Optional) | 01 (feet below ground surface) |

Notes:

(1) The Sample Number is a unique, 8-digit number assigned to each sample.

(2) The Sample Name or Location ID is an optional identifier that can be used to further describe each sample or sample location.

Table 2. Sampling and Analysis - Young America Mine Tailings Impoundment Removal Assessment 10KN

| Data Quality | Sampling Area | Matrix | Sampling Pattern | Sample Type | Data Quality | Number of Field Samples | Analyte or Parameter | Method Number | Action Level | Method Quant. Limit | #/type of Sample Containers per Sample | Preservative | Hold Time | Field QC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field Screen | Grid | Soil | Grid | Grab | Screening | ~35 | XRF | SOP | Pb As Zn Cu Cd | Pb As Zn Cu Cd | Baggie | none | 6 months | 2% Duplicate |
| Lab Analysis | Highest lead | Soil | Targeted | Grab | Definitive | 3 | Total Metals | 6010 |  |  | 8 oz jar | none | 6 months |  |
| Lab Analysis | Drinking water wells | Water | Targeted | Grab | Definitive | 2 | Total Metals | 200.8 |  |  | 1L poly | pH <2 HNO3 | 6 months | 1 Duplicate 1 Blank |
| Lab Analysis | Highest lead | Soil | Targeted | Grab | Definitive | 3 | SPLP | 1312/200.8 |  |  | 8 oz jar | none | 6 months |  |
| Lab Analysis | Highest lead | Soil | Targeted | Grab | Definitive | 2 | TCLP | 1311/6010 |  |  | 8 oz jar | none | 6 months |  |

Note: For matrix spike and/or duplicate samples, no extra volume is required for a soil sample. Double volume is required for an inorganic water sample.

Table 3. Common Sample Handling Information

| Analysis Type | Sub Analysis | Matrix | Analytical Method | Container Type | Minimum Volume | Preservative | Temperature/ Storage | Hold Time | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Metals | Metals Not including Mercury or Hexachrome. Includes TAL, PP, RCRA lists) | Solid | EPA 6000 / 7000 Series | Glass Jar | 200 g | n/a | None | 6 months | SW-846 ch. 3 |
|  |  | Aqueous | EPA 6000 / 7000 Series | PTFE or HDPE | 600 mL | HNO3 to pH < 2 | Not listed | 6 months | SW-846 ch. 3 |
|  | Mercury | Solid | EPA 7471B | Glass Jar | 200 g | n/a | < 6o C | 28 days | SW-846 ch. 3 |
|  |  | Aqueous | EPA 7470A | PTFE or HDPE | 400 mL | HNO3 to pH < 2 | Not listed | 28 days | SW-846 ch. 3 |
|  | Hexavalent Chromium, (Hexachrome, Cr+6) | Solid | Lab-specific soil extraction modification, EPA 7196A | Glass Jar | 100 g | n/a | < 6o C | 28 days to extraction | SW-846 ch. 3 |
|  |  | Aqueous | EPA 218.6 (Drinking Water) | PTFE or HDPE | 400 mL | n/a | < 6o C | 24 hours | SW-846 ch. 3 |
|  | XRF | Solid (in situ; on the ground surface) | 6200 | none | n/a | none | none | Analyze Immediately | n/a |
|  |  | Solid (ex situ) | 6200 | plastic bag | 200 g | none | none | 6 months | n/a |
| VOCs | VOCs / BTEX | Solid | EPA 5035 / 8260B | * | * | * | * | 2 days to lab / 14 days | SW-846 ch. 4 |
|  |  | Aqueous | EPA 8260B | Amber Vial with Septa Lid | 2 x 40 mL | HCl to pH< 2 | < 6o C (headspace free) | 14 days | SW-846 ch. 4 |
| SVOCs | SVOCs / PAHs | Solid | EPA 8270D | Glass Jar | 8 ounces | n/a | < 6o C | 14 days | SW-846 ch. 4 |
|  |  | Aqueous | EPA 8270D | Amber Glass | 2 x 1 L | n/a | < 6o C | 7 days | SW-846 ch. 4 |
| PCBs and Dioxins/Furans | PCBs | Solid | EPA 8082 | Glass Jar | 8 ounces | n/a | < 6o C | none | SW-846 ch. 4 |
|  |  | Aqueous | EPA 8082 | Amber Glass | 2 x 1 L | n/a | < 6o C | none | SW-846 ch. 4 |
|  | Dioxins/Furans | Solid | EPA 8280 or 8290 | Glass Jar | 8 ounces | n/a | < 6o C | none | SW-846 ch. 4 |
|  |  | Aqueous | EPA 8280 or 8290 | Amber Glass | 2 x 1 L | n/a | < 6o C | none | SW-846 ch. 4 |
| Pesticides and Herbicides | Chlorinated Pesticides | Solid | EPA 8081 | Glass Jar | 8 ounces | n/a | < 6o C | 14 days | SW-846 ch. 4 |
|  |  | Aqueous | EPA 8081 | Amber Glass | 2 x 1 L | n/a | < 6o C | 7 days | SW-846 ch. 4 |
|  | Chlorinated Herbicides | Solid | EPA 8151 | Glass Jar | 8 ounces | n/a | < 6o C | 14 days | SW-846 ch. 4 |
|  |  | Aqueous | EPA 8151 | Amber Glass | 2 x 1 L | n/a | < 6o C | 7 days | SW-846 ch. 4 |
| NWTPH | Gasoline-Range Organics | Solid | TPHs/NWTPH-Gx | Amber Glass Jar with Septa Lid | 4 ounces | n/a | < 6o C (headspace free) | 14 days | Method |
|  |  | Aqueous | TPHs/NWTPH-Gx | Amber Vial with Septa Lid | 2 x 40 mL | pH < 2 with HCl | < 6o C (headspace free) | 7 days unpreserved 14 days preserved | Method |
|  | Diesel-Range Organics | Solid | 3510, 3540/3550, 8000 | Glass Jar | 8 ounces | n/a | < 6o C | 14 days | Method |
|  |  | Aqueous | 3510, 3540/3550, 8000 | Glass Amber | 2 x 1 L | pH < 2 with HCl | < 6o C | 7 days unpreserved 14 days preserved | Method |
| Geotechnical | Particle Size Analysis | Solid | ASTM D-422 | Glass Jar or Plastic Bag | 2 x 8 ounce | none | n/a | n/a | Method |
| Miscellaneous | pH | Solid | EPA 9045 | Glass Jar | 8 ounces | n/a | n/a | Analyze Immediately | SW-846 ch. 3 |
|  |  | Aqueous | EPA 9040 | PTFE | 25 mL | n/a | n/a | Analyze Immediately | SW-846 ch. 3 |
|  | Total Organic Carbon (TOC) | Solid | SW-846 9060 | Glass Jar | 100 mL | n/a | < 6o C | 28 days | SW-846 |
|  |  | Aqueous | EPA 415.1 | PTFE or HDPE | 200 mL | store in dark HCL or H2SO4 to pH <2 | < 6o C | 7 days unpreserved 28 days preserved | Method |
|  | Cyanide | Solid | SW-846 9013 | Glass Jar | 5 g | n/a | < 6o C | 14 days | SW-846 ch. 3 |
|  |  | Aqueous | SW-846 9010C | PTFE or HDPE | 500 mL | NaOH to pH > 12 | < 6o C | 14 days | SW-846 ch. 3 |
|  | Conductivity | Aqueous | EPA 120.1 | PTFE or HDPE | 100 mL | n/a | n/a | Analyze Immediately | Method |
|  | Hardness | Aqueous | EPA 130.1 | PTFE or HDPE | 1 x 1 L | HNO3 to pH<2 | < 6o C | 28 days | Method |
|  | Total Suspended Solids | Aqueous | EPA 160.2 | PTFE or HDPE | 100 mL | n/a | < 6o C | 7 days | Method |
|  | Total Dissolved Solids | Aqueous | EPA 160.1 | PTFE or HDPE | 100 mL | n/a | < 6o C | 7 days | Method |
|  | Nitrate/nitrite | Aqueous | EPA 353.2 | PTFE or HDPE | 1 x 250 mL | H2SO4 to pH <2 | < 6o C | 28 days | Method |
|  | Nitrate | Aqueous | SW-846 9210A | PTFE or HDPE | 1,000 mL | n/a | < 6o C | 28 days | SW-846 ch. 3 |
|  | Nitrite | Aqueous | SW-846 9216 | PTFE or HDPE | 25 mL | n/a | < 6o C | 48 hours | SW-846 ch. 3, Method |
|  | Fluoride | Aqueous | SW-846 9214 | PTFE or HDPE | 300 mL | n/a | < 6o C | 28 days | SW-846 ch. 3 |
|  | Chloride | Aqueous | SW-846 9250 | PTFE or HDPE | 50 mL | n/a | < 6o C | 28 days | SW-846 ch. 3 |
|  | Sulfate | Aqueous | SW-846 9035 | PTFE or HDPE | 50 mL | n/a | < 6o C | 28 days | SW-846 ch. 3 |
|  | Sulfide | Solid | SW-846 9215 | Glass Jar | 1 x 4 ounces | Fill sample surface with 2N zinc acetate until moistened. | < 6o C (headspace free) | 7 days | SW-846 ch. 3 |
|  |  | Aqueous | SW-846 9031 | PTFE or HDPE | 100 mL | 4 drops 2N zinc acetate/100 mL sample; NaOH to pH>9. | < 6o C (headspace free) | 7 days | SW-846 ch. 3 |

Key:

| * | = See individual methods. We typically collect 3xEnCore-type samplers and 1x40 mL VOA vial per sample, keep at < 6oC with no chemical preservative, and they must be at the lab within 48 hours of collection. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| C | = Celsius | HNO3 | = nitric acid | SVOCs | = semivolatile organic compounds |
| Cr | = chromium | L | = liter | SW-846 | = EPA Test Methods for Evaluating Solid Waste, Physical/Chemical Methods |
| EPA | = Environmental Protection Agency | mL | = milliliter | TAL | = Target Analyte List |
| g | =grams | n/a | = not applicable | TPH | = total petroleum hydrocarbons |
| H2SO4 | = sulfuric acid | NaOH | = sodium hydroxide | VOA | = Volatile Organic Analysis |
| HCL | = hydrochloric acid | PCBs | = polychlorinated biphenyls | VOCs | = Volatile Organic Compounds |
| HDPE | = high-density polyethylene | PTFE | = polytetrafluoroethylene |  |  |
| Hg | = mercury | RCRA | = Resource Conservation and Recovery Act |  |  |